# COURSERA HANDS-ON LEARNING PROJECT REPORT

SystemVerilog Tutorials: Hardware Design & Verification Practice

# SCALABLE MULTI-OPERATION FPGA CALCULATOR IP

DESIGN SPECIFICATION & VERIFICATION REPORT

## METADATA & PROJECT SUMMARY

| - Author / Student: | - Hoàng Ngọc Gia Bão |
| --- | --- |
| - Course Module: | - Hands-on-Learning: Core Architecture & Module Foundations |
| - Target Hardware Architecture: | - IEEE 1800 SystemVerilog RTL on Intel Quartus Prime FPGA |
| - Verification Environment: | - QuestaSim / ModelSim Automated Coverage Suite (Self-Checking Testbench) |
| - Status: | - Fully Verified (100% Code & Functional Coverage) |

# TABLE OF CONTENTS

METADATA & PROJECT SUMMARY	1

TABLE OF CONTENTS	2

1. Overview	3

1.1. Introduction & Background	3

1.2. Key Architectural Features	3

1.3. System Block Diagram	3

1.4. Interface Signal Specification	4

2. Architectural & Functional Specification	5

2.1. Top-Level Controller & Handshake Protocol (calculator_top)	5

2.2. Dual-Port Asynchronous Register Bank (storage_unit)	5

2.3. Extended Precision Combinational ALU Core (alu_core)	5

2.4. Zero-Division Hardware Exception Protection	6

3. Verification Strategy & Testbench Framework	6

3.1. Self-Checking Verification Architecture	6

3.2. Stimulus Generation & Corner Case Strategy	6

3.3. QuestaSim Timing Waveform Verification	7

4. Coverage Analysis & Quality Metrics	7

4.1. Coverage Closure Results (100% Target vs. Actual)	7

4.2. Hierarchical Instance Coverage Verification	8

5. Build Automation & Reproducibility (Makefile)	9

6. Document Revision History	10

# 1. Overview

## 1.1. Introduction & Background

Arithmetic compute engines form the backbone of modern digital signal processors (DSP), microcontrollers, and application-specific accelerators. This design document presents the specification, architecture, and verification results of a parameterizable, synthesizable multi-operation arithmetic calculator IP implemented in SystemVerilog (IEEE 1800).

Originating from the Coursera project "Hands-on-Learning: Core Architecture & Module Foundations", this work significantly expands the original high-level architectural plan into a complete, tapeout-ready hardware implementation featuring dedicated register-file memory staging, zero-stall dual-bus operand retrieval, and a fully automated, self-checking verification suite achieving 100% Code Coverage.

## 1.2. Key Architectural Features

- Modular Multi-Module Partitioning: Decoupled storage (storage_unit) and combinational arithmetic processing (alu_core), orchestrated under a single-cycle registered pipeline controller (calculator_top).

- Fully Parameterizable Datapath: Generic configuration parameters (DATA_WIDTH = 16, OUT_WIDTH = 32, NUM_REGS = 4) allowing flexible adaptation across diverse FPGA platforms.

- Dual-Port Asynchronous Read Access: Independent combinational read multiplexers provide zero-latency concurrent access to two operands simultaneously without memory pipeline stalls.

- Signed Two's Complement Precision: Native support for 16-bit signed operands with an extended 32-bit output datapath to eliminate overflow during wide multiplications and additions.

- Deterministic Hardware Exception Trapping: Combinational zero-divisor comparator triggers dedicated status flags (div_by_zero_flag) and immediately clamps outputs to zero, protecting downstream systems from invalid states.

- Industrial-Grade Verification Closure: Automated QuestaSim testbench with randomized corner cases achieving 100% Statement, Branch, Condition, Toggle, and Assertion coverage.

## 1.3. System Block Diagram

The top-level calculator encapsulates an internal dual-bus storage bank and an arithmetic logic core. Communication is handled synchronously with registered valid-flag handshaking.

Figure 1: Scalable FPGA Calculator Architecture Diagram

## 1.4. Interface Signal Specification

| Signal Name | Width | Direction | Clock Domain | Reset State | Functional Description |
| --- | --- | --- | --- | --- | --- |
| clk | 1 | Input | System | N/A | Primary system clock reference (50 MHz, 20 ns period). |
| rst_n | 1 | Input | Asynchronous | Active-Low | System reset. Resets internal registers and flags to 0. |
| wr_en | 1 | Input | clk (sync) | N/A | Synchronous write enable strobe for the internal register bank. |
| wr_addr | 2 | Input | clk (sync) | N/A | Target register destination address (2'b00 to 2'b11). |
| data_in | 16 | Input | clk (sync) | N/A | 16-bit signed input data to be stored into the targeted register. |
| rd_addr_a | 2 | Input | Combinational | N/A | Source address pointer for Operand A. |
| rd_addr_b | 2 | Input | Combinational | N/A | Source address pointer for Operand B. |
| opcode | 3 | Input | clk (sync) | N/A | Operation selection code (NOP, ADD, SUB, MUL, DIV). |
| op_valid | 1 | Input | clk (sync) | N/A | Instruction trigger strobe indicating valid inputs. |
| data_out | 32 | Output | clk (sync) | 32'sh0000_0000 | Registered 32-bit signed computational result. |
| result_valid | 1 | Output | clk (sync) | 1'b0 | Active-high validity flag. Asserts 1 cycle after op_valid. |
| div_by_zero_flag | 1 | Output | clk (sync) | 1'b0 | Active-high hardware exception flag indicating zero denominator. |

# 2. Architectural & Functional Specification

## 2.1. Top-Level Controller & Handshake Protocol (calculator_top)

The top-level module functions as a single-cycle registered pipeline coordinator:

1. Data Ingestion: When wr_en is asserted at the rising edge of clk, data_in is loaded into the register indexed by wr_addr.

2. Concurrent Fetch: Address lines rd_addr_a and rd_addr_b continuously route the internal stored contents directly into the ALU input ports via combinational decoding.

3. Execution & Staging: When op_valid is asserted, alu_core evaluates the arithmetic result combinatorially. On the next rising clock edge, data_out latches the evaluated data, while result_valid asserts to notify the receiver.

## 2.2. Dual-Port Asynchronous Register Bank (storage_unit)

The storage unit contains an internal array declared as:

logic signed [DATA_WIDTH-1:0] reg_bank [0:NUM_REGS-1];

- Synchronous Write: A single write port commits data strictly on posedge clk when wr_en = 1 and rst_n = 1.

- Combinational Dual-Bus Read: Two separate output buses operate independently:

assign rd_data_a = reg_bank[rd_addr_a];
assign rd_data_b = reg_bank[rd_addr_b];

This architecture guarantees that the execution unit receives both operands instantaneously without consuming additional clock cycles for memory read handshakes.

## 2.3. Extended Precision Combinational ALU Core (alu_core)

The execution unit operates purely combinationally. To eliminate arithmetic truncation and handle signed 2's complement representations accurately, all 16-bit operands are sign-extended to 32 bits before operation execution.

| Opcode [2:0] | Mnemonic | Hardware RTL Implementation | Hardware Exception Trigger |
| --- | --- | --- | --- |
| 3'b000 | NOP | alu_result = 32'sh0; zero_div_err = 1'b0; | None |
| 3'b001 | ADD | alu_result = 32'(operand_a) + 32'(operand_b); | None |
| 3'b010 | SUB | alu_result = 32'(operand_a) - 32'(operand_b); | None |
| 3'b011 | MUL | alu_result = 32'(operand_a) * 32'(operand_b); | None (Mapped to FPGA DSP blocks) |
| 3'b100 | DIV | alu_result = (operand_b == 0) ? 0 : 32'(operand_a / operand_b); | operand_b == 16'sh0000 |
| Others | DEFAULT | alu_result = 32'sh0; zero_div_err = 1'b0; | Safe State Clamping |

## 2.4. Zero-Division Hardware Exception Protection

When an operation requests division (opcode = 3'b100) and operand_b equals zero:

1. The combinational comparator immediately drives zero_div_err = 1'b1 and forces alu_result = 32'sh0.

2. At the subsequent rising clock edge, calculator_top synchronously asserts div_by_zero_flag <= 1'b1 and latches data_out <= 32'sh0000_0000.

3. This fail-safe mechanism prevents hardware deadlock and ensures deterministic behavior across all corner cases.

# 3. Verification Strategy & Testbench Framework

## 3.1. Self-Checking Verification Architecture

The test environment (tb/tb_calculator_top.sv) applies modern verification methodologies utilizing reusable automatic tasks and assertions:

- apply_reset(): Enforces asynchronous reset assertion for 3 clock periods and verifies register bank initialization.

- write_reg(addr, val): Drives data ingestion cleanly at negedge clk with setup/hold margin protection.

- run_op(op, a, b, exp_out, exp_err): Stimulates arithmetic instructions, measures pipeline latency, and checks actual responses against a golden reference model using SystemVerilog assertions:

assert (result_valid === 1'b1) else $error("[FAIL] Valid deasserted!");
assert (data_out === exp_out && div_by_zero_flag === exp_err)
    else $error("[FAIL] Result mismatch! Expected: %0d, Got: %0d", exp_out, data_out);

## 3.2. Stimulus Generation & Corner Case Strategy

The test suite in tc/tc_basic_arithmetic.sv is divided into two distinct functional phases:

- Phase 1: Datapath Toggle Exhaustion: Alternating bit-patterns (0x5555, 0xAAAA, 0xFFFF, 0x0000) are sequentially loaded into registers R0-R3. This guarantees that every individual bit wire on both read multiplexers transitions through both 0 → 1 and 1 → 0 states.

- Phase 2: Comprehensive Arithmetic & Exception Verification: Evaluates signed combinations including:

- Addition: 100 + (-25) = 75

- Subtraction: 100 - (-25) = 125

- Multiplication: (-25) × 300 = -7500 (32-bit product generation)

- Division: 100 / (-25) = -4

- Division by Zero: 100 / 0 = 0 with div_by_zero_flag = 1

## 3.3. QuestaSim Timing Waveform Verification

The timing diagram illustrates synchronous register ingestion, dual-operand fetch, back-to-back arithmetic execution, and proper 1-cycle pipeline delay on result_valid.

Figure 2: QuestaSim Simulation Timing Waveform

# 4. Coverage Analysis & Quality Metrics

## 4.1. Coverage Closure Results (100% Target vs. Actual)

The verification suite achieved 100.00% Coverage Closure across all structural and functional domains.

| Coverage Metric | Industry Standard Target | Total Bins | Bins Hit | Misses | Final Achievement |
| --- | --- | --- | --- | --- | --- |
| Statement Coverage | 100% | 92 | 92 | 0 | 100.00% (PASS) |
| Branch Coverage | 100% | 14 | 14 | 0 | 100.00% (PASS) |
| Toggle Coverage | 100% | 454 | 454 | 0 | 100.00% (PASS) |
| Functional Covergroups | 100% | 7 | 7 | 0 | 100.00% (PASS) |
| Assertion Coverage | 100% | 2 | 2 | 0 | 100.00% (PASS) |

## 4.2. Hierarchical Instance Coverage Verification

Coverage database analysis confirms that 100% coverage was achieved across all sub-modules (u_storage, u_alu, and calculator_top) without excluding any design logic.

Figure 3: QuestaSim Overall Coverage Dashboard

Figure 4: Hierarchical Instance Coverage Tree & Recursive Bins

# 5. Build Automation & Reproducibility (Makefile)

The entire design compilation, simulation, database merging, and reporting workflow is encapsulated in an automated Makefile:

| Makefile Target | Action & Description |
| --- | --- |
| make clean | Removes all temporary files, simulation logs, and UCDB databases. |
| make all_cov | Compiles RTL and TB with +cover=bcesft enabled and executes batch simulation. |
| make gen_cov | Merges simulation coverage databases into a unified IP.ucdb and outputs text reports. |
| make gen_html | Generates interactive HTML coverage dashboards. |
| make view_cov | Opens QuestaSim GUI in -viewcov mode to inspect coverage trees natively. |
| make wave | Launches waveform viewer pre-configured with wave.do color formatting. |

# 6. Document Revision History

| Revision | Release Date | Summary of Changes | Author |
| --- | --- | --- | --- |
| v1.0 | 2026-09-28 | Initial architectural planning submission for Coursera module. | Hoàng Ngọc Gia Bão |
| v2.0 | 2026-09-29 | Implemented full SystemVerilog RTL and self-checking testbench. | Hoàng Ngọc Gia Bão |
| v3.0 | 2026-09-30 | Achieved 100% coverage closure, timing verification, and finalized report. | Hoàng Ngọc Gia Bão |